LV-BST-H2.56Aa

Household Energy Storage

Capacity

51.2V 50Ah Single module

Smart Household Energy Storage System

- High-voltage LiFePO4 battery solution, Single module is 51.2V 50Ah 2.56kWh.

- 3 to 10 layers recommended.

- Inverter you need to choose 380V 3phase input and output, common is 8-15KW inverter.

- Cobalt Free Lithium Iron Phosphate (LFP) Battery: Maximum Safety, Life Cycle and Power.

- Applicable on grid or hybrid on and off-Grid solar energy storage system.

- Self-Consumption Optimization for Residential and Commercial Applications.

- Modular Design Simplifies Transport and Installation.

Field power

supply

Farm

Household

Application Scenarios

Nomadic area

Communication base station

Villa

Advantages

- High Voltage

- High Capacity

- Scalable Design

- Integrated Design

- Efficient Performance

- Safe and Reliable

- Real-time Monitoring

- Intelligent BMS

| Technical Specification |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | 153.6V 50Ah | 204.8V 50Ah | 256V 50Ah | 307.2V 50Ah | 358.4V 50Ah | 409.6V 50Ah | 460.8V 50Ah | 512V 50Ah |
| Number of layers | 3 layers | 4 layers | 5 layers | 6 layers | 7 layers | 8 layers | 9layers | 10layers |
| Picture |  |  |  |  |  |  |  |  |
| Energy | 7.68KWh | 10.24KWh | 12.8KWh | 15.36KWh | 17.92KWh | 20.48KWh | 23.04KWh | 25.6KWh |
| Operating Voltage Range | 129.6V～168.48V | 172.8V～224.64V | 216V～280.8V | 259.2V～336.96V | 302.4V～393.12V | 345.6～449.28V | 388.8V～505.44V | 432V～561.6V |
| Dimension (L*W*H) | 600*210*820 | 600*210*980 | 600*210*1140 | 600*210*1300 | 600*210*1460 | 600*210*1620 | 600*210*1780 | 600*210*1940 |
| Net Weight | 102.5 | 129 | 155.5 | 182 | 208.5 | 235 | 261.5 | 288 |
| Recommend charge current |  | 10~25A |  |  |  |  |  |  |
| Max continue charge current |  | 50A |  |  |  |  |  |  |

| Max continue discharge current | 50A |
| --- | --- |
| Peak current | 100A |
| Display | The information of Battery, such as SOC, battery voltage and so on |
| Communication | Support RS485 / CAN |
| Operating temperature | -20℃～55℃ |
| Environment | Indoor |
| Relative humidity | 5%～95% |
| Cooling | Natural convection |
| Cell technology | Lithium-iron phosphate (LiFePO4) |
| Life cycle | 6000 times @80%DOD |
| Single module Technical Specification |  |
| Dimension (L*W*H) | High Voltage Box:600*210*250 Battery Box:600*210*160 |
| Battery module weight | 26.5kg |